Received 4 February 2025, accepted 24 February 2025, date of publication 3 March 2025, date of current version 14 March 2025.

Digital Object Identifier 10.1109/ACCESS.2025.3547430

IEEE Access

Multidisciplinary Rapid Review Open Access Journal

RESEARCH ARTICLE

Automatic Brain Tumor Segmentation: Advancing U-Net With ResNet50 Encoder for Precise Medical Image Analysis

İD

SHOFFAN SAIFULLAH 1,2, RAFAŁ DREŻEWSKIR1, ANTON YUDHANA3,

AND ANDIKO PUTRO SURYOTOMO

ID 2

1Faculty of Computer Science, AGH University of Krakow, 30-059 Krakow, Poland

2Department of Informatics, Universitas Pembangunan Nasional Veteran Yogyakarta, Yogyakarta 55281, Indonesia

3 Department of Electrical Engineering, Universitas Ahmad Dahlan, Yogyakarta 55191, Indonesia

Corresponding author: Shoffan Saifullah (saifulla@agh.edu.pl)

This work was supported in part by the "Excellence Initiative-Research University" Program (AGH University of Krakow), PLGrid, under Grant PLG/2023/016757; in part by Ahmad Dahlan University under Grant U12/191/II/2024; in part by STER NAWA under Project 12.12.455.00710; and in part by the Polish Ministry of Science and Higher Education funds assigned to AGH University of Krakow.

ABSTRACT Automated brain tumor segmentation from MRI images is critical for accurate diagnosis and treatment planning. This study presents a novel ResUNet50-based approach, integrating ResNet50 as an encoder within the U-Net framework to achieve robust and precise segmentation. The proposed model was evaluated on two datasets: a Kaggle-based T1-CE MRI dataset and BraTS 2018, ensuring comprehensive assessment across different imaging conditions. ResUNet50 outperformed state-of-the-art models, achieving Dice coefficients exceeding 0.95 and Jaccard indices above 0.91 on the Kaggle dataset. Additionally, exper- iments on BraTS 2018 Whole Tumor segmentation across multiple MRI modalities (FLAIR, T1, T1-CE, and T2) demonstrated high accuracy on both High-Grade Gliomas (HGG) and Low-Grade Gliomas (LGG), confirming model generalization. Statistical significance tests (Paired t-tests and Wilcoxon Signed-Rank Tests, p < 0.05) validated the improvements over existing approaches. Furthermore, ResUNet50 reduced parameters by over 60% and accelerated inference time by 4.8x compared to U-Net, enhancing its potential for clinical deployment. Future work will focus on ensemble learning and bio-inspired optimization for improved robustness across multi-center MRI datasets. Explainable AI techniques such as Grad-CAM and saliency maps will be incorporated to enhance interpretability, improving clinical applicability.

INDEX TERMS Brain tumor segmentation, ResUNet50, medical image analysis, MRI scans deep learning.

I. INTRODUCTION

Brain tumor segmentation plays a pivotal role in the diagnosis and treatment planning of neurological disorders [1], [2]. Accurate delineation of tumor regions from medical images, such as magnetic resonance imaging (MRI) scans [3], is crucial for assessing tumor size, location, and progres- sion [4]. However, radiologists' manual segmentation is time-consuming and prone to subjective interpretation and interobserver variability [5], [6], [7]. As the volume and com- plexity of medical imaging data continue to increase, there is

The associate editor coordinating the review of this manuscript and approving it for publication was Ganesh Naik

VOLUME 13, 2025

ID

a growing need for automated segmentation techniques that can efficiently and reliably analyze such data [8], [9], [10].

Deep learning-based approaches have emerged as promis- ing solutions for automated medical image segmentation tasks [2], [11]. Among these approaches, the U-Net architec- ture has garnered significant attention due to its effectiveness in capturing spatial context and preserving fine details in segmented regions [12], [13], [14]. The U-Net architecture comprises an encoder-decoder [15] structure with skip connections, allowing the integration of high-resolution features from different levels of abstraction [16]. Despite its success in various medical imaging applications [17], [18], traditional U-Net models may struggle to capture intricate

© 2025 The Authors. This work is licensed under a Creative Commons Attribution 4.0 License. For more information, see https://creativecommons.org/licenses/by/4.0/

43473

IEEE Access

S. Saifullah et al.: Automatic Brain Tumor Segmentation: Advancing U-Net With ResNet50 Encoder

details and subtle variations in complex structures, such as brain tumors [19], [20].

To address this challenge [21], researchers have explored the integration of advanced convolutional neural network (CNN) architectures as encoders in the U-Net frame- work [22], [23], [24], [25]. Among these architectures, ResNet50 stands out for its superior performance in fea- ture extraction tasks and the ability to handle deeper networks without suffering from vanishing gradients [26], [27]. By leveraging ResNet50 as an encoder, the resulting hybrid model, referred to as ResUNet50, aims to enhance the segmentation performance of U-Net while maintaining computational efficiency.

This article presents a comprehensive analysis of the ResUNet50 model for automating brain tumor segmentation in MRI scans. We delve into the architectural details of both U-Net and ResNet50, elucidating their respective strengths and weaknesses in medical image analysis. We discuss the rationale behind integrating ResNet50 as an encoder in the U-Net framework and outline the methodology used to train and evaluate the ResUNet50 model. Furthermore, we present experimental results demonstrating the efficacy of ResUNet50 in accurately delineating tumor regions from MRI scans. Through in-depth analysis and comparison with existing approaches, we highlight the potential of ResUNet50 to advance the field of medical image segmentation and facilitate more precise diagnosis and treatment planning for patients with brain tumors.

II. RELATED WORKS

Automated segmentation of brain tumors represents a crucial and challenging task in medical image analysis [2]. Histori- cally, segmentation techniques relied on handcrafted features and classical machine learning algorithms such as support vector machines (SVM) and random forests [28]. These early methods, which required intensive preprocessing steps [29] such as intensity normalization and skull stripping, showed promising results for specific datasets but often failed to generalize across diverse imaging modalities and tumor types due to the reliance on manually engineered features [30].

With the advent of deep learning, particularly convolu- tional neural networks (CNNs), a paradigm shift occurred in medical image segmentation [30]. The introduction of the U-Net architecture by Ronneberger et al. [31] marked a significant development due to its ability to efficiently capture spatial context and preserve detailed information through skip connections, making it a popular choice for medical segmentation tasks [32].

Recent advances have brought various improvements to the basic U-Net framework to address specific challenges in brain tumor segmentation [17], [33]. Further improvement can be achieved by integrating attention mechanisms with U- Net-based models [34], focusing computational resources on relevant regions to improve Accuracy and robustness [35], [36]. Additionally, the adoption of dense and residual connections, inspired by DenseNet and ResNet architectures,

43474

respectively, has facilitated feature reuse and supported the training of deeper networks, enhancing the models' ability to learn complex representations without degrading gradient flow [37], [38].

Further refinements included the adaptation of pre-trained CNNs for medical imaging [39]. Transfer learning strategies were used to utilize networks pre-trained on large natural image datasets, adapting them to the medical domain with minimal fine-tuning [40], [41]. This approach has proven effective in enhancing segmentation performance, particularly in scenarios characterized by limited annotated medical images [42].

Despite these advancements, the field continues to face significant challenges, such as dataset imbalance, noise and artifacts in medical images, and variability in tumor appearance [43], [44]. These issues complicate the training process and affect the generalization of the models to new patient data [45]. Innovative approaches have been proposed to overcome these hurdles, such as the incorporation of Edge U-Net [23], which utilizes edge detection to refine segmentation boundaries, and ensemble models leveraging multimodal data.

Recent research has also focused on integrating more sophisticated architectures such as ResUNet, which combines residual connections with the U-Net structure to allow deeper feature extraction and mitigate the vanishing gradient problem [46], [47]. In addition, emerging techniques such as graph convolutional networks [48] and attention-based models are being explored to improve feature representation and spatial coherence, aiming to enhance both the Accuracy and the clinical relevance of segmentation outputs [49].

Despite these improvements, key limitations persist in existing U-Net variants, including (1) limited generalization to diverse tumor morphologies, (2) ineffective handling of small or irregular tumor regions, (3) sensitivity to dataset imbalance, and (4) challenges in maintaining seg- mentation accuracy across datasets with varying contrast and noise levels [50], [51]. While approaches such as Edge U-Net and ensemble-based segmentation methods help refine boundaries and improve generalization, they often introduce additional computational overhead and may still struggle with cases involving highly heterogeneous tumors. These challenges necessitate a more efficient and adaptable solution.

To address these issues, we propose ResUNet50, a hybrid model that integrates ResNet50 within the U-Net framework to enhance segmentation accuracy, feature extraction, and robustness across diverse patient populations. ResNet50's residual connections improve gradient flow and deep feature learning, mitigating the vanishing gradient problem com- mon in deeper networks. Additionally, its optimized skip connections enable better spatial detail retention, allowing for precise tumor boundary delineation. This integration enhances generalization, reduces over-segmentation errors, and ensures more accurate segmentation even in cases of small and irregular tumor structures.

VOLUME 13, 2025

S. Saifullah et al.: Automatic Brain Tumor Segmentation: Advancing U-Net With ResNet50 Encoder

IEEE Access

III. MATERIALS AND METHODS

This section outlines the methodologies adopted in this study, including the Brain MRI dataset preparation, the integration of PSO-based preprocessing, the modified U-Net architecture with a ResNet50 encoder, and the evaluation metrics used to assess segmentation performance. The proposed approach leverages bio-inspired optimization techniques to refine segmentation quality, optimize thresholding and morphological refinements dynamically, and enhance test- time augmentation, ensuring robustness across various MRI scans. Figure 1 illustrates the complete workflow of the proposed method, detailing each step from preprocessing to final segmentation output.

Input Data

MRI dataset:

Kaggle (3 classes)

BraTS 2018 (4 modalities)

Preprocessing

- Resizing

Grayscale Conversion Standardization

PSO-Optimized Preprocessing

Threshold Selection

- Morphological Refinement

Deep Learning Modeling

-U-Net-ResNet50 encoder

Training evaluation

- Inference (prediction)

PSO-Optimized Test-Time Augmentation (TTA)

- Augmentations

- Flips & Rotations PSO-based Selection

Post-Processing PSO-Optimized Morphological Refinement - Noise Removal, Gap Filli-

ng. Final Refinement

Final Segmentation

- Segmentation Masks - Metrics Results - Final Masks

FIGURE 1. Workflow diagram of the proposed approach, illustrating the complete segmentation pipeline from preprocessing, PSO-optimized thresholding, U-Net-based modeling, test-time augmentation, and final post-processing.

A. BRAIN MRI DATASET AND PREPROCESSING

Our research utilizes a specialized dataset from Kaggle, sourced from the T1-CE MRI collection, which includes 3064 brain MRI images [52] essential for training our tumor segmentation model. Each image maintains a high-quality standard, featuring a resolution of 512 x 512 pixels and a 24-bit color depth, which is vital for capturing detailed visual information necessary for precise tumor analysis. A key component of our methodology is the inclusion of accurate ground truth tumor masks for each image, which is fundamental to our supervised learning approach.

The images are categorized into three tumor types: Meningioma (708 images), Glioma (1426 images), and Pituitary (930 images). This classification not only showcases the diversity within brain tumor types but also enhances the adaptability and effectiveness of our model. Figure 2 illustrates this classification with examples of original MRI images, their associated tumor masks, highlighting the complexity of the dataset and the unique challenges associated with each type of tumor that our model was specifically designed to address.

VOLUME 13, 2025

(a)

(b)

(c)

FIGURE 2. Sample dataset of brain MRI images with their ground truth (Mask) of (a) Meningioma, (b) Glioma, and (c) Pituitary.

Meningiomas are typically benign but can be aggressive (Grade II-III), potentially causing vision impairment and neurological deficits due to brain compression [53]. Gliomas, ranging from low-grade to highly malignant glioblastoma multiforme (GBM), are highly infiltrative, with GBM having a poor prognosis and a median survival of less than 15 months, making precise boundary delineation crucial for treatment planning [54], [55]. Pituitary tumors, often benign adenomas, impact hormone regulation, potentially leading to metabolic disorders such as Cushing's disease and acromegaly, while macroadenomas can compress the optic chiasm, resulting in vision loss [56]. The severity of these tumors necessitates robust segmentation methods, where meningiomas require structural contouring, gliomas demand precise tumor margin detection, and pituitary tumors need accurate localization.

To further evaluate the generalization ability of ResUNet50, we conducted experiments using the BraTS 2018 dataset [57], [58], [59], a well-established benchmark for brain tumor segmentation. Unlike the Kaggle dataset, which consists of single-modality T1-CE images, BraTS 2018 includes multi-modal MRI scans (FLAIR, T1, T1-CE, and T2), capturing both High-Grade Gliomas (HGG) and Low-Grade Gliomas (LGG). This dataset provides tumor masks for Whole Tumor (WT) segmentation, encompassing edema, non-enhancing tumor, and enhancing tumor regions, making it highly relevant for clinical applications. The use of multiple MRI modalities ensures that our model can effectively learn from diverse intensity distributions and structural variations. Figure 3 presents sample images of HGG and LGG tumors across different MRI modalities along with their corresponding ground truth masks.

Our preprocessing routine is tailored to optimize image quality for effective segmentation, employing essential tech- niques to standardize the images and enhance both computa- tional efficiency and segmentation Accuracy [5], [60]:

•

Image Resizing: The first preprocessing step involves resizing the images to 256 x 256 pixels. This change is crucial for standardizing data across the dataset, reducing memory consumption, and enhancing the speed of our computational processes. It provides a vital

43475

IEEE Access

S. Saifullah et al.: Automatic Brain Tumor Segmentation: Advancing U-Net With ResNet50 Encoder

High-Grade Gliomas (HGG)

Low-Grade Gliomas (LGG)

(a)

(b)

(c)

(d)

(e)

FIGURE 3. Sample dataset from BraTS 2018 with multi-modal MRI scans: (a) FLAIR, (b) T1, (c) T1-CE, (d) T2, and (e) Ground Truth (Whole Tumor/WT).

balance between retaining sufficient image detail for accurate analysis and maintaining manageable image sizes for efficient processing.

Depth Reduction: After resizing, the images are con- verted from their original 24-bit color depth to 8-bit grayscale. This conversion shifts the focus of our model to intensity variations, which are more relevant for tumor identification than color information. It signifi- cantly reduces the computational load by minimizing memory usage and simplifies the processing workflow. Additionally, this step improves compatibility with various datasets and streamlines further preprocessing and enhancement techniques.

These preprocessing steps are critical for enhancing the Accuracy and efficiency of our segmentation model. By ensuring consistency in image processing, we facilitate a more streamlined segmentation workflow and enhance the model's generalization capabilities across different types of brain tumors. These measures are crucial for enabling reliable model performance in practical clinical applications and set the stage for detailed analyzes and experimental evaluations that follow in our study.

B. PSO-OPTIMIZED PREPROCESSING FOR

THRESHOLDING AND MORPHOLOGICAL REFINEMENT Thresholding is a crucial step in segmentation, as it deter- mines how predicted probability maps are converted into binary tumor masks [61]. Traditional methods often use a fixed threshold (e.g., 0.5), but this approach fails to account for contrast variations across different MRI scans, leading to suboptimal segmentation results [62]. Instead of using a static threshold, we apply PSO to determine the optimal binarization threshold for each dataset dynamically [63], [64]. In this optimization framework, each particle in PSO represents a candidate threshold value, ranging between. 0.1 and 0.9, and the algorithm iteratively adjusts these values to maximize the Dice Coefficient between predicted and ground truth masks. The Dice coefficient is computed as Eq. 1.

43476

LDice

2Σytrueypred+€ Σytrue + Σypred +ε

(1)

where ytrue represents the ground truth mask, ypred represents the binarized model prediction, and € is a small smoothing constant to prevent division errors. The PSO algorithm iteratively searches for the optimal threshold (*) that maximizes Dice similarity, ensuring that tumor regions are accurately segmented while minimizing false positives and negatives.

Even with optimized thresholding, segmentation masks may still contain small noise artifacts, incomplete tumor regions, or disconnected tumor structures, necessitating further refinement. To address these issues, we apply mor- phological post-processing, which involves operations such as opening, closing, and dilation to enhance mask quality. However, selecting the appropriate morphological operation and its kernel size is a non-trivial task that traditionally requires manual tuning. Instead, we use PSO to select the best morphological transformation and kernel size automatically. Each particle in PSO represents a unique combination of kernel size (k) and operation type (o), where the operation can be opening (for noise removal), closing (to fill gaps), or dilation (to expand tumor regions). The fitness function evaluates each combination based on Dice similarity, and the optimal parameters are selected dynamically.

In addition to preprocessing and post-processing optimiza- tions, we further enhance segmentation robustness through Test-Time Augmentation (TTA). During inference, models can produce inconsistent predictions depending on image transformations such as flips, rotations, and scaling. To mit- igate this issue, we apply PSO to optimize TTA strategies, ensuring that the best set of augmentations is used during testing. Each particle in PSO represents a unique combination of augmentation parameters, including horizontal flipping, vertical flipping, and rotational transformations (0°, 90°, 180°, 270°). The fitness function evaluates segmentation Accuracy under each augmentation scenario, and the final prediction is obtained by averaging the best-performing augmented outputs. This approach significantly improves segmentation stability across MRI scans.

C. MODIFIED U-NET WITH RESNET50 FOR MEDICAL

IMAGE SEGMENTATION

The U-Net architecture [12], [63], initially developed for biomedical image segmentation, is renowned for its effi- ciency and effectiveness, primarily due to its symmetric structure and skip connections, which simultaneously facil- itate precise localization and detailed context. Building on this robust foundation, our study introduces a significant modification by integrating the ResNet50 architecture [60] as the encoder within the U-Net framework (Figure 4). This modification is designed to harness the strengths of ResNet50, particularly its deep residual learning framework, to enhance feature extraction capabilities and improve the model's ability to learn from complex medical imaging data.

1) RESNET50 AS ENCODER

ResNet50, a variant of the Residual Network architecture, introduces residual learning to address the degradation

VOLUME 13, 2025

S. Saifullah et al.: Automatic Brain Tumor Segmentation: Advancing U-Net With ResNet50 Encoder

IEEE Access

Input

Image

Mask

conv_block

256x256x3

128x128x128

64x64x256

32x32x128

1

Con2D Batch_Norm Activation

encoder block ResNet50

ResNet50 Encoder

Batch Norm ReLU Conv2D(3x3)

Shortcut

decoder block

Conv2DTranspose Concate (skip_feature) conv_block

256x256x2

Output

Mask

Predicted

max pool 2x2 copy & crop conv 2x2

conv 1x1

Batch Norm ReLU

Conv2D(3x3)

Conv2D(1x1)

Skip

Connections

Decoder Block

FIGURE 4. The architecture of modified U-Net with ResNet50 Encoder (Proposed ResUNet50).

problem inherent in training deeper networks [65]. By incor- porating shortcut connections that skip one or more layers, ResNet50 allows the training of substantially deeper net- works by alleviating the vanishing gradient problem. When utilized as an encoder in the U-Net architecture, ResNet50 enables the propagation of gradients through many layers, enhancing feature extraction without significantly increasing the computational burden. The deep features extracted by the ResNet50 encoder contain rich contextual information and subtle details crucial for differentiating regions of interest in medical images, such as tumorous tissue from normal tissues.

2) U-NET ARCHITECTURE ENHANCEMENT

In our proposed U-Net-based model, the encoder path has been enhanced by replacing it with a ResNet50 backbone. Unlike typical U-Net structures, we introduce modifications to the convolutional blocks within both the encoder and decoder paths. Each convolution block in the ResNet50 encoder has been modified with a 2x configuration, as depicted in the figure, allowing for double the depth of feature extraction at every layer. This modification facilitates deeper feature representation without significantly increasing computational complexity, ensuring more robust segmentation performance.

In the decoder path, the model retains the traditional U-Net structure but is enhanced with up-sampling layers and optimized concatenations of the corresponding encoded features. This ensures that the high-level, abstract features extracted during the downsampling phase are effectively utilized during the upsampling phase, resulting in more precise segmentation [66].

Additionally, we have fine-tuned the skip connections, which are crucial for reintroducing spatial and structural information lost during the encoding phase. These connec- tions directly concatenate feature maps from the encoder to the decoder and apply a dynamic weighting mechanism to prioritize the most relevant features at each resolution level.

VOLUME 13, 2025

This strategy ensures that both low-level spatial information and high-level semantic features are effectively used for the accurate localization and segmentation of brain tumors.

Furthermore, our approach modifies the convolutional layers within the decoder, incorporating specialized 2x con- figurations to allow for richer feature refinement as the spatial resolution increases. The introduction of this 2x configuration within both the encoder and the decoder improves the model's ability to capture and retain important tumor features, leading to enhanced segmentation Accuracy.

Finally, our method integrates advanced preprocessing techniques for MRI contrast enhancement, which improves the model's performance [64] across varied MRI conditions in the Kaggle dataset. This preprocessing phase is crucial to ensure that the model can generalize well to a variety of patient data, providing more reliable segmentation results for various types of brain tumors.

3) INTEGRATION AND TRAINING

Integrating ResNet50 into U-Net involves adjusting the standard U-Net architecture to accommodate the depth and complexity of ResNet50's output. This requires careful calibration of the network layers to ensure that the depth of feature maps matches across the encoder and decoder paths. Training this integrated model involves a fine-tuning process in which the pre-trained weights of ResNet50 (trained on large-scale image datasets like ImageNet) are adapted to specific tasks of medical image segmentation [67]. This transfer learning approach leverages the generalized feature detection capabilities of ResNet50, enhancing the model's performance, especially when dealing with limited medical imaging data.

4) EVALUATION AND APPLICATION

The effectiveness of this modified U-Net with ResNet50 encoder is evaluated based on its ability to segment medical images accurately. The incorporation of deep residual learning significantly boosts the model's sensitivity and specificity, making it particularly adept at handling medical images where tumor boundaries are not distinctly defined. By leveraging the advanced capabilities of both U-Net and ResNet50, our model represents a powerful tool for medical image analysis, promising substantial improvements over traditional segmentation methods in terms of Accuracy, reliability, and clinical applicability.

D. EVALUATION MEASURES

In this section, we delve into the evaluation metrics [12], [13] utilized to gauge the performance of our modified U-Net with ResNet50 for medical image segmentation. These metrics are quantitative indicators of Accuracy, con- sistency, and similarity between the predicted segmentation masks and ground truth annotations [63]. To provide a comprehensive assessment of the model's segmentation performance, we incorporated traditional evaluation metrics

43477

IEEE Access

S. Saifullah et al.: Automatic Brain Tumor Segmentation: Advancing U-Net With ResNet50 Encoder

such as Accuracy, Loss Function, Dice Coefficient, and Jaccard Index, alongside supplementary metrics like Pixel Accuracy, Mean Pixel Accuracy, Precision, Recall, F1 Score, and Mean IoU.

•

•

Accuracy (ACC) is a fundamental metric that quantifies the proportion of correctly classified pixels in the segmentation mask compared to the ground truth. It provides insights into the correctness of the model's predictions, encompassing both true positive and true negative rates. A high accuracy score indicates that the model effectively distinguishes between tumor and non-tumor regions in the images. The Accuracy is calculated using Eq. 2.

ACC =

TP + TN

TP+TN+FP+ FN

(2)

where TP (True Positives) represents correctly seg- mented tumor pixels, TN (True Negatives) represents correctly identified non-tumor pixels, FP (False Pos- itives) represents misclassified tumor pixels, and FN (False Negatives) represents misclassified non-tumor pixels.

Pixel Accuracy (PA): Pixel Accuracy represents the percentage of correctly classified pixels across the entire dataset, offering a global view of model performance. However, it may not effectively capture class imbal- ance, particularly when tumor regions are significantly smaller than the background. It is calculated similarly to Accuracy but provides an overall pixel-wise assessment. Pixel Accuracy is expressed by Eq. 3.

PA

=

i=1

TPi

(TP; + FP; + FN; + TN;)

where N is the number of classes.

that are correctly identified by the model. A high recall value indicates that the model successfully detects tumor regions with minimal omission of critical structures. This is particularly important in clinical applications where missing a small portion of a tumor could impact diagnosis and treatment. However, recall alone does not indicate precision, meaning a model with very high recall may still produce false positives by over-segmenting non-tumor areas. Recall is computed as Eq. 6.

Recall =

TP

TP+FN

(6)

F1 Score: The F1 Score is the harmonic mean of Precision and Recall. It balances both false positives and false negatives, providing a single measure of the overall segmentation quality of the model. It is particularly useful when there is a trade-off between precision and recall. The F1 Score is expressed as Eq. 7

F1

=

2 x Precision x Recall

Precision + Recall

(7)

• The Loss function quantifies the discrepancy between the predicted segmentation mask and the ground truth annotations, guiding the optimization process toward accurate segmentation. It penalizes incorrect classi- fications at the pixel level, ensuring precise tumor boundary delineation. The Binary Cross-Entropy (BCE) Loss, commonly used in medical image segmentation, is defined as Eq. 8.

N

1

Loss

(3)

N

. Mean Pixel Accuracy (MPA): Mean Pixel Accuracy addresses class imbalance by computing the average accuracy across each class (tumor and non-tumor). It ensures that the performance of the model in smaller classes, such as tumor regions, is given equal importance as the background. This metric (Eq. 4) is especially critical in medical segmentation, where class imbalance is common.

Σ[y; log(yi) + (1 − y;) log(1 − ŷ;)] (8)

i=1

where N is the total number of pixels. This function min- imizes classification errors and improves segmentation

accuracy.

The Dice Coefficient measures the spatial overlap between the predicted and ground truth segmentation masks, offering a more sensitive evaluation metric for imbalanced datasets. It quantifies the similarity between two sets by calculating the ratio of twice the intersection of the sets to the sum of their total elements. The Dice coefficient is expressed as Eq. 9.

N

MPA

=

1

N

Σ

TPi

TPi+FNi

(4)

Dice

=

2 x |AUB

|A|+|B|

(9)

• Precision: Precision measures the proportion of cor- rectly predicted tumor pixels out of all predicted tumor pixels. It focuses on minimizing false positives, which is particularly important in clinical settings to avoid over-segmentation or misdiagnosis. Precision is calculated as Eq. 5.

Precision =

TP

TP + FP

(5)

where |X| and |Y| represent the predicted and ground truth masks, respectively.

• Jaccard Index (JI) quantifies the similarity between the predicted and ground truth segmentation masks by computing the ratio of their intersection to their union. (IoU). It provides a measure of spatial overlap similar to the Dice coefficient but emphasizes the shared region relative to the total area. The Jaccard index is defined as Eq. 10.

• Recall (Sensitivity): Recall, also referred to as Sensi- tivity, quantifies the proportion of actual tumor pixels

43478

JAN B JI =

JAU BI

(10)

VOLUME 13, 2025

S. Saifullah et al.: Automatic Brain Tumor Segmentation: Advancing U-Net With ResNet50 Encoder

IEEE Access

• Mean Intersection over Union (Mean IoU): Mean IOU provides a more detailed evaluation by averaging the IoU across all classes (Eq. 11, ensuring that both tumor and non-tumor regions are accurately evaluated. Mean IoU is particularly valuable in measuring the boundary delineation Accuracy, critical for medical applications.

Mean IOU =

1

N

JA; Bil JA; U Bil

(11)

In conclusion, these evaluation measures offer comprehen- sive insights into the performance of our segmentation model, considering Accuracy, Loss, spatial overlap, and similarity between predicted and ground truth masks. By leveraging these metrics, we ensure robust evaluation and validation of our model's effectiveness in medical image segmentation tasks, ultimately advancing the field of computer-aided diagnosis and treatment planning for patients with brain

tumors.

IV. RESULTS AND DISCUSSIONS

This section presents the results of our experiments and engages in a detailed discussion. We begin by outlining the implementation process and experimental setup used to evaluate the proposed ResUNet50 model for brain tumor seg- mentation. Then, we present the quantitative and qualitative results, analyze the evaluation measures used, and compare our method with existing state-of-the-art approaches.

A. EXPERIMENTAL SETUP AND IMPLEMENTATION DETAILS

Our experiments, detailed in Table 1, were conducted on a high-performance computing setup with 8 NVIDIA A100- SXM4-40GB GPUs, using a Jupyter Notebook environment with TensorFlow and Keras. We developed a modified U-Net architecture integrated with a ResNet50 encoder to enhance depth and feature extraction capabilities. The ADAM optimizer was used with a learning rate of 0.001, and training was set for 50 epochs with early stopping based on validation Loss.

TABLE 1. Summary of the experimental setup.

across all samples. Unlike many deep learning-based seg- mentation methods, we did not employ data augmentation in this study. This decision was made to preserve the integrity of the tumor structures, as artificial transformations (such as flips, rotations, and intensity variations) could alter essential tumor features, potentially impacting segmentation accuracy. Additionally, our dataset already included diverse tumor shape, size, and intensity variations, which mitigated the risk of overfitting.

Each batch was processed with 64 samples, balancing computational efficiency and training stability. To ensure a balanced distribution of all tumor types (Meningioma, Glioma, and Pituitary) across the training and testing datasets, we employed a stratified split. Stratification was performed to maintain the proportional representation of each tumor type, preventing any class imbalance that could bias the model toward more prevalent tumor categories. This approach ensures that the model learns effectively from all tumor types and can generalize to unseen data. The dataset was split into a 0.8:0.2 ratio for training and testing.

To assess model performance, we monitored Accuracy and Loss during training and utilized the Dice coefficient and Jac- card index for segmentation evaluation. These metrics were chosen for their ability to quantify the spatial overlap between predicted segmentation maps and ground truth, providing a robust assessment of the model's precision in delineating tumor boundaries for clinical applications. Future work will explore the potential impact of augmentation strategies, such as elastic deformations, intensity scaling, and affine transformations, to further enhance segmentation robustness, particularly for small or irregular tumor structures.

B. IMPACT OF PSO-BASED PREPROCESSING AND

POST-PROCESSING

To evaluate the effectiveness of PSO-driven thresholding, morphological post-processing, and TTA, we compared seg- mentation performance before and after applying PSO opti- mization. The results demonstrate that PSO-based enhance- ments lead to significant improvements in segmentation Accuracy.

TABLE 2. Effect of PSO on threshold and morphology optimization.

Parameter

GPUs

Environment

Value

8 NVIDIA A100-SXM4-40GB

Jupyter Notebook with TensorFlow and Keras Modified U-Net with ResNet50 encoder

Architecture

Optimizer

ADAM

Learning Rate

0.001

Epochs

50

256x256 pixels, 3 channels

Metric Dice Score

Before PSO After PSO Improvement (pp)

0.9107

0.9298

+1.91

IoU Score

0.8374

0.8690

+3.16

Precision Recall

0.8690

0.8879

+1.89

1.0000

0.9883

-1.17

Image Size

Data Augmentation

Batch Size

Train-Test Split Evaluation Metrics

Not used

64

0.8:0.2 (stratified by tumor type) Accuracy, Loss, Dice Coefficient, Jaccard Index

MRI scans were standardized to 256x256 pixels with three channels, normalized to the range [0,1], and skull-stripped to remove non-brain regions, ensuring spatial consistency

VOLUME 13, 2025

The results indicate that PSO-threshold optimization led to a 1.91 percentage point (pp) improvement in the Dice coefficient, demonstrating that adaptive threshold selection enhances segmentation Accuracy. Similarly, IoU increased by 3.16 pp, suggesting that tumor boundaries were more accurately delineated after optimization. Mor- phological refinement using PSO improved precision by 1.89 pp, as noise artifacts were effectively removed from the

43479

IEEE Access

S. Saifullah et al.: Automatic Brain Tumor Segmentation: Advancing U-Net With ResNet50 Encoder

segmentation masks. However, recall decreased slightly by 1.17 pp, indicating that some minor tumor regions were suppressed during post-processing.

These findings confirm that PSO-driven preprocessing and post-processing significantly enhance segmentation performance. By dynamically optimizing threshold values, morphological operations, and augmentation strategies, PSO ensures that segmentation masks are adaptively refined for each MRI scan, leading to more accurate and clinically relevant predictions. Integrating PSO into the preprocessing pipeline reduces manual parameter tuning, making the seg- mentation framework more robust and generalizable. Future research could explore extending PSO-based optimization to 3D MRI segmentation and incorporating multi-objective optimization techniques to improve segmentation perfor- mance while minimizing computational overhead.

Accuracy

1.0000

0.9975

0.9950

0.9925

0.9900

0.9875

0.9850

0.9825

0.05

0.04

Loss

0.03

Training and validation Accuracy

A

0

10

-9

Training Accuracy Validation Accuracy

20

Epochs

(a)

30

40

50

Training and validation Loss

Training Loss Validation Loss

C. EVALUATION AND SEGMENTATION RESULTS This section comprehensively evaluates our proposed method for brain tumor segmentation, analyzing classification and segmentation performance using multiple metrics. The eval- uation framework includes Accuracy, Loss, Dice coefficient, Jaccard index, and additional segmentation-specific metrics such as Pixel Accuracy (PA), Mean Pixel Accuracy (MPA), Precision, Recall, F1 Score, and Mean IoU.

To assess the learning behavior of our model, we moni- tored key performance metrics throughout the training and validation phases. Figure 5 illustrates the performance curves for Accuracy, Loss, Dice Coefficient, and Jaccard Index, highlighting the model's convergence and stability.

Accuracy serves as a fundamental metric for evaluating classification performance. Our models demonstrated excep- tional Accuracy values across the training and validation datasets, surpassing 99.8% and 99.9% in the training and validation datasets, respectively. The consistency of the Accuracy values across different classes underscores the robustness and generalizability of our models. Furthermore, the absence of outliers in the Accuracy distribution reinforces the stability and reliability of our classification results.

Loss metrics provide insights into our models' training dynamics and convergence stability. We observed low Loss values across all classes, ranging from 0.0020 to 0.0025, indi- cating minimal classification errors during model training. The narrow interquartile ranges (IQRS) of Loss highlight our training procedures' consistency and convergence stability. Additionally, the absence of outliers in the Loss distribution underscores the robustness of our training methodologies.

The Dice coefficient is critical for evaluating the Accuracy of segmentation algorithms in delineating tumor regions. Our analysis revealed consistently high Dice coefficients across all classes, ranging from 0.9091 to 0.9576. These results indicate substantial agreement between predicted and ground truth tumor segmentations, emphasizing the Accuracy and precision of our segmentation algorithms. Moreover, the absence of outliers in the Dice coefficient distribution further

Dice

Jaccard

0.02

0.01

0.00

0

10

20

30

40

50

Epochs

(b)

0.9

0.8

0.7

0.6

0.5

0.4

0.3

Training and validation Dice

Training Dice Validation Dice

0.2

0

10

20

30

40

50

Epochs

(c)

0.9

0.8

0.7

0.6

0.5

0.4

0.3

0.2

0.1

0

Training and validation Jaccard

-9

Training Jaccard Validation Jaccard

10

20

30

40

50

Epochs

(d)

FIGURE 5. Evaluation measures of training (red line) and Validation (blue line) of our proposed method: (a) Accuracy, (b) Loss, (c) Dice coefficient, and (d) Jaccard index.

validates the reliability and consistency of our segmentation results.

43480

VOLUME 13, 2025

S. Saifullah et al.: Automatic Brain Tumor Segmentation: Advancing U-Net With ResNet50 Encoder

The Jaccard coefficient offers insights into the overlap between predicted and ground truth tumor regions, providing additional perspectives on segmentation quality. We observed high median Jaccard coefficients across all classes, exceeding 0.8355 in the training dataset and 0.8517 in the validation dataset. The narrow IQRS and absence of outliers in the Jaccard coefficient distribution underscore the Accuracy and reliability of our segmentation techniques, highlighting their potential clinical utility in aiding diagnosis and treatment planning.

The consistency of the model's predictions across different tumor types is further analyzed in Figure 6. Figure 6 presents box plots depicting the distribution of evaluation measures across different classes, including Accuracy, Loss, Dice Coefficient, and Jaccard Index. These plots offer insights into the variability and performance of the model across diverse tumor types, providing valuable insights for further analysis and interpretation.

The exemplary performance of our proposed methods across various evaluation measures underscores their efficacy and potential clinical impact on brain tumor analysis. High Accuracy and low Loss metrics reflect our models' superior predictive capability and convergence stability during training. Furthermore, the consistently high Dice and Jaccard coefficients affirm the Accuracy and reliability of our segmentation algorithms in delineating tumor regions with precision.

In addition to traditional metrics such as Accuracy, Loss, Dice Coefficient, and Jaccard Index, we incorporated several supplementary metrics, including Pixel Accuracy (PA), Mean Pixel Accuracy (MPA), Precision, Recall, F1 Score, and Mean IoU, to provide a more detailed and comprehensive assessment of our model's segmentation performance, with the results shown in Table 3.

TABLE 3. Evaluation of the effectiveness of the proposed model in brain tumor segmentation using selected metrics.

Values

Values

Values

Values

0.99920

0.99915

0.99910

0.99905

0.99900

0.99895

0.99890

0.0025

0.0024

0.0023

0.0022

0.0021

0.0020

0.96

0.95

0.94

0.93

T

Accuracy

Training

(a)

Loss

Training

(b)

Dice

0.92

0.91

Training

(c)

0.92

0.90

0.88

Jaccard

IEEE Access

Validation

Validation

Validation

Metric

Value

Accuracy (ACC)

99.9%

Pixel Accuracy (PA)

98.2%

Mean Pixel Accuracy (MPA)

95.4%

Precision

93.5%

Recall (Sensitivity)

94.5%

F1 Score

94.0%

92.1%

95.7%

85.17%

Mean IOU

Dice Coefficient

Jaccard Index (IoU)

While Pixel Accuracy (98.2%) and Mean Pixel Accuracy (95.4%) are high, it is important to recognize that Pixel Accuracy can be misleading in imbalanced datasets, such as medical imaging scenarios where tumor regions occupy a relatively small proportion of the image. Since Pixel Accuracy calculates the proportion of correctly classified pixels across the entire image, it may overestimate segmen- tation performance if the model correctly classifies large background regions while failing to segment tumor areas accurately. For instance, in an image where the tumor

VOLUME 13, 2025

0.86

0.84

Training

(d)

Validation

FIGURE 6. Box plots depict (a) Accuracy, (b) Loss, (c) Dice coefficient, and (d) Jaccard index of variations across classes, Offering insights into the model's evaluation measures.

comprises only 10% of the total pixels, a naive model that classifies all pixels as background could still achieve 90% Pixel Accuracy despite failing to segment any part of the tumor.

To address this limitation, we emphasize the Dice Coef- ficient (95.7%) and Jaccard Index (85.17%), more reliable indicators of segmentation quality in imbalanced datasets. The Dice Coefficient, which quantifies the spatial overlap between predicted and ground truth tumor regions, ensures

43481

IEEE Access

S. Saifullah et al.: Automatic Brain Tumor Segmentation: Advancing U-Net With ResNet50 Encoder

that false positives and false negatives are accounted for in the evaluation. Similarly, the Jaccard Index (also known as Intersection over Union, IoU) is a stricter measure of overlap, focusing on the proportion of correctly segmented tumor pixels relative to the combined predicted and actual tumor regions. These metrics provide a more clinically relevant assessment of segmentation accuracy, as they directly measure tumor boundary precision, which is crucial for accurate diagnosis and treatment planning.

A key strength of our model is the high recall (94.5%), which has significant clinical relevance in reducing under- segmentation risks. Under-segmentation, where portions of the tumor are missed, poses a significant risk in clinical practice, as it can lead to incomplete diagnosis and subop- timal treatment planning. A high recall ensures that nearly all tumor pixels are correctly detected, reducing the likelihood of undiagnosed tumor regions. This is particularly crucial for radiotherapy planning, where precise tumor volume estimation is essential, as well as for surgical navigation and disease progression monitoring.

While high recall minimizes false negatives (missed tumor regions), precision (93.5%) is equally important to avoid over-segmentation, where non-tumor regions are misclassified as tumor tissue. Over-segmentation may lead to unnecessary interventions, excessive surgical resection, or misguidance in tumor volume estimation. Thus, balancing recall and precision is essential for accurate tumor detection while preventing misclassification of healthy brain tissue. The F1 Score (94.0%), which harmonizes precision and recall, confirms that our method achieves an optimal trade- off, ensuring high tumor detection rates while maintaining specificity.

Furthermore, Mean IoU (92.1%), Dice Coefficient (95.7%), and Jaccard Index (85.17%) indicate that our model achieves robust overlap with ground truth masks, demonstrating its effectiveness in capturing complex tumor structures. These high values confirm that the model preserves detailed tumor boundaries, essential for precise volumetric analysis and clinical decision-making.

To further validate our model's performance, Figure 7 presents visual comparisons between ground truth masks and predicted segmentation for each tumor type. Each class exhibits distinct characteristics, influencing the segmentation outcomes and emphasizing the need for tailored evaluation and interpretation.

The exceptional performance observed for the Menin- gioma and Glioma classes, characterized by Dice coefficients of 0.9716 and 0.9703, respectively, underscores the model's proficiency in accurately delineating tumor regions. These high coefficients signify a remarkable agreement between the predicted and ground truth tumor segmentations, reflecting the model's robustness and precision.

Conversely, the slightly future performance observed for the Pituitary class, with Dice coefficient of 0.9459, needs closer examination. Pituitary tumors present inherent challenges due to their diverse morphological characteristics

43482

(a)

(b)

(c)

(d)

Meningioma:

Dice=0.9716

Jaccard=0.9448

Glioma:

Dice=0.9703 Jaccard=0.9423

Pituitary:

Dice=0.9459

Jaccard=0.8974

FIGURE 7. Segmentation results of (a) Brain MRI, (b) Ground truth (Mask), (c) Predictions, and (d) the overlap based on each class.

(a)

(b)

Meningioma

Glioma

Putitary

FIGURE 8. Visualization of predicted and ground truth masks using (a) error maps to highlight segmentation differences and (b) attention heatmaps to illustrate the model's focus regions.

and proximity to critical brain structures. Despite this com- plexity, our model demonstrates commendable segmentation Accuracy, albeit with a marginally reduced Dice coefficient compared to other classes.

Figure 7 provides visual representations of the segmen- tation results, offering invaluable insights into the model's performance at a qualitative level. Through brain MRI images, ground truth masks, model predictions, and overlay visualizations, clinicians gain a holistic understanding of seg- mentation outcomes, facilitating informed decision-making in clinical practice.

While these segmentation results demonstrate high perfor- mance across tumor classes, further analysis of segmentation boundaries and model focus regions is necessary to assess limitations. To achieve this, we generated error maps and attention heatmaps to examine subtle differences between ground truth and predicted masks, as well as highlight key areas where the model focuses during segmentation. Figure 8 visualizes these aspects.

The error maps in Figure 8(a) highlight minor discrep- ancies between ground truth and predicted masks. These visualizations confirm that the model accurately captures tumor boundaries with few misclassified regions. Unlike conventional models prone to over-segmentation (false pos- itives) or under-segmentation (false negatives), our model demonstrates high consistency in delineating tumor areas without excessive inclusion of non-tumor structures.

VOLUME 13, 2025

S. Saifullah et al.: Automatic Brain Tumor Segmentation: Advancing U-Net With ResNet50 Encoder

IEEE Access

From a clinical perspective, accurate boundary delineation is crucial for ensuring that tumor margins are precisely identified, which is essential in surgical planning and radiotherapy dose estimation. The minimal segmentation errors observed suggest that the model mitigates common challenges such as boundary misalignment and small region under-segmentation.

The attention heatmaps in Figure 8(b) illustrate the regions where the model focuses during segmentation. The sub- stantial overlap between ground truth and predicted regions confirms that the model learns the correct tumor localization without significant misclassification. Importantly, there is no evidence of model distraction toward non-tumor areas, indi- cating that the segmentation approach effectively suppresses irrelevant background information.

Although the results show high segmentation accuracy, further refinements could enhance the model's robustness:

Refining Boundary Detection: Enhancing tumor edge precision by incorporating boundary-aware loss func- tions or contour-preserving mechanisms.

•

•

Improving Small Tumor Region Detection: Fine-tuning multi-scale feature extraction methods to ensure optimal performance on less prominent tumor areas.

. Enhancing Model Interpretability: Further exploration of uncertainty maps could provide deeper insights into model confidence across different tumor structures. The inclusion of error maps and attention heatmaps provides qualitative validation of the model's segmentation accuracy. The strong alignment between ground truth and predictions confirms that the proposed method achieves high precision in tumor localization, effectively capturing tumor regions without over-segmentation or misclassifica- tion. These insights support the clinical applicability of the model in brain tumor diagnosis and treatment planning, demonstrating its reliability for real-world medical imaging tasks.

D. COMPARISON WITH STATE-OF-THE-ART METHODS In this section, we thoroughly compare our segmentation results with existing state-of-the-art methods for brain tumor segmentation. Our evaluation focuses on assessing the efficacy of various approaches in accurately delineating tumor regions within MRI images. We utilize widely accepted metrics to quantify segmentation performance, namely the Dice coefficient and Jaccard Index, which measure the similarity between predicted and ground truth segmentation masks.

Our comparative analysis reveals compelling insights into the performance of various segmentation methods across different tumor classes and all images. Table 4 provides a comprehensive overview of the segmentation efficacy achieved by each method, presenting the Dice coefficients and Jaccard indices for Meningioma, Glioma, Pituitary, and overall segmentation across all tumor classes.

When evaluating Meningioma segmentation, our proposed method demonstrates outstanding performance, achieving

VOLUME 13, 2025

TABLE 4. Comparison of segmentation efficacy for each class and all images using various approaches.

Methods

Proposed Method CNN-based [68] Edge U-Net [23] U-Net with AT [69] U-Net modality [70] QuickTumorNet [71] Proposed Method CNN-based [68] Edge U-Net [23] U-Net with AT [69] U-Net modality [70] QuickTumorNet [71] Proposed Method CNN-based [68] Edge U-Net [23] U-Net with AT [69] U-Net modality [70] QuickTumorNet [55] Proposed Method CNN-based [68] U-Net with AT [69] SegNet [72] DFP-Unet [73] Residual-Unet [74] Resnet101-U-Net [75]

Class

Dice

Jaccard

0.9575

0.9191

0.894

Meningioma 0.888

0.7985

0.8243

0.9459

0.801

0.9585

0.9207

0.779

Glioma

0.9176 0.8478

0.6077

0.7241

0.64

0.9187 0.8517

0.813

Pituitary

0.8728 0.7743

0.7847

0.9108

0.812

0.9553 0.9151

0.828

0.6239

All Images

0.9332 0.8827

0.8169

0.9011

0.8369 0.85

QuickTumorNet [71]

0.724

DeepLab with Pre-trained [76] Cascaded dual-scale LinkNet [77] SegNet VGG16 [78]

0.8091

0.8003

0.9314 0.7622

Dice coefficient of 0.9575 and Jaccard Index of 0.9191. These results surpass those obtained by several existing methods, including CNN-based [68], Edge U-Net [23], U-Net with Adaptive Thresholding [69], U-Net modality [70], and QuickTumorNet [71]. Similarly, for Glioma segmentation, our method excels with Dice coefficient of 0.9585 and Jac- card Index of 0.9207, outperforming competing approaches such as CNN-based [68], Edge U-Net [23], U-Net with Adaptive Thresholding [69], U-Net modality [70], and QuickTumorNet [71].

For Pituitary segmentation, our method achieves notable results with Dice coefficient of 0.9187 and Jaccard Index of 0.8517, showcasing its superiority over CNN-based [68], Edge U-Net [23], U-Net with Adaptive Thresholding [69], U-Net modality [70], and QuickTumorNet [71].

Moreover, when considering segmentation across all tumor classes (All Images), our proposed method consistently delivers superior performance, achieving Dice coefficient of 0.9553 and Jaccard Index of 0.9151. These results out- shine various existing methods, including CNN-based [68], U-Net with Adaptive Thresholding [69], SegNet [72], DFP-Unet [73], Residual-Unet [74], Resnet101-U-Net [75], and Quick TumorNet [71].

The superior performance of ResUNet50 can be attributed to several key architectural advancements. First, the inte- gration of residual learning mitigates the vanishing gradient problem encountered in deep CNN architectures. Traditional segmentation models, such as Edge U-Net and QuickTu- morNet, struggle with deep feature extraction due to limited

43483

IEEE Access

S. Saifullah et al.: Automatic Brain Tumor Segmentation: Advancing U-Net With ResNet50 Encoder

residual connections, leading to a loss of critical spatial infor- mation. By contrast, ResUNet50 employs residual blocks that facilitate information flow across layers, preserving fine structural details and enabling deeper networks to learn richer feature representations. This results in enhanced tumor boundary delineation and improved segmentation accuracy.

Another significant advantage of ResUNet50 is its opti- mized skip connections, which enhance spatial refinement. Standard U-Net architectures rely on basic skip connections to transfer low-level spatial features; however, ResUNet50 enhances this mechanism by incorporating deep feature fusion from residual blocks, allowing for better integration of high- and low-level representations. Compared to Edge U-Net, which primarily focuses on boundary enhancement, ResUNet50 achieves superior segmentation consistency by preserving both spatial and contextual coherence. This leads to higher Dice scores, particularly in complex tumor morphologies.

Moreover, ResUNet50 demonstrates strong generalization capabilities across different tumor types, addressing a major limitation of previous methods such as U-Net with Adaptive Thresholding and QuickTumorNet, which struggle with dataset-specific performance variations. These methods often suffer from fixed parameter tuning, limiting their adaptability across different imaging conditions. In contrast, ResUNet50 incorporates multi-scale feature learning, allowing it to generalize effectively across diverse tumor structures and achieve consistent segmentation accuracy.

For small tumor detection, ResUNet50 also provides a notable improvement over existing models. Pituitary tumors, which are challenging to segment due to their small size and irregular morphology, are often misclassified in standard CNN-based models. Our results indicate that ResUNet50 achieves a Dice coefficient of 0.9187 for Pituitary tumors, significantly surpassing Edge U-Net (0.8728) and Quick- TumorNet (0.812). This demonstrates that our architecture excels in detecting smaller tumor regions while maintaining high segmentation precision.

Additionally, ResUNet50 outperforms advanced models such as DeepLab with Pre-trained MobileNetv2 [76] (Dice: 0.8091, Jaccard: 0.8254), Cascaded dual-scale LinkNet [77] (Dice: 0.8003, Jaccard: 0.9074), and SegNet VGG16 [78] (Dice: 0.9314, Jaccard: 0.7622). While these models are designed to handle multi-scale features and general seg- mentation tasks, our modified U-Net with a ResNet50 backbone and optimized skip connections enables more accurate feature extraction and integration, particularly for complex brain tumor structures. This results in higher Dice and Jaccard values, indicating superior segmentation perfor- mance, especially in capturing fine details and boundaries of tumor regions.

Another key advantage of our approach is the reduction. of false positives and over-segmentation errors, a major concern in prior CNN-based methods. Many existing seg- mentation models tend to over-segment tumor regions, leading to high false-positive rates and reduced clinical

43484

reliability. ResUNet50 effectively balances precision and recall, reducing misclassification errors while maintaining high sensitivity, as reflected in its high Jaccard Index (0.9151). This improvement is particularly beneficial in clinical applications, where reducing false positives is crucial for minimizing unnecessary interventions and enhancing diagnostic confidence.

While the proposed segmentation method demonstrates superior performance in terms of the Dice coefficient and Jaccard index, assessing whether these improvements are statistically significant is essential. To validate the robustness of our approach, we conducted paired t-tests and Wilcoxon signed-rank tests, comparing our method with state-of-the- art segmentation techniques. These statistical tests help. determine whether the observed differences in performance metrics arise from meaningful improvements rather than random variations.

The paired t-test evaluates whether the mean difference between paired segmentation results is statistically signifi- cant. The test statistic is given by Eq. 12.

t =

Sal√n

(12)

where d is the mean difference between paired values, sa is the standard deviation of differences, and n is the number of samples. A p-value (p < 0.05) indicates a statistically significant difference.

The Wilcoxon signed-rank test, a non-parametric alterna- tive, evaluates the differences in ranks, given by Eq. 13.

W = ΣR+

(13)

where R+ represents the sum of positive rank differences. A low p-value (p < 0.05) confirms that the performance difference is statistically meaningful.

Figure 9 presents the statistical significance results for Dice and Jaccard indices using the paired t-test and Wilcoxon signed-rank test. The results confirm that all comparisons between our proposed method and existing segmentation methods are statistically significant (p < 0.05). Notably, methods such as DeepLab Pre-trained (p 0.0029), ResNet101-U-Net (p

0.0065), and Edge U-Net (p = 0.0034) exhibit moderate statistical significance comp- ared to simpler architectures like QuickTumorNet and U-Net with Adaptive Thresholding, which demonstrate highly significant p-values (p < 0.001).

=

=

The statistical analysis further supports the superiority of the ResUNet50 architecture, which integrates residual learning, optimized skip connections, and multi-scale feature extraction. These enhancements contribute to significantly improved segmentation accuracy across different tumor types. The consistently low p-values validate that our model's advancements in feature preservation, boundary delineation, and robust segmentation are not due to chance but reflect meaningful performance improvements.

These findings indicate that our proposed method sig- nificantly outperforms existing segmentation approaches,

VOLUME 13, 2025

S. Saifullah et al.: Automatic Brain Tumor Segmentation: Advancing U-Net With ResNet50 Encoder

IEEE Access

p-values

10-2

10-3-

Statistical Significance of Proposed Method vs. State-of-the-Art Methods

CNN-based

Edge U-Net

U-Net with AT

U-Net modality

QuickTumorNet

SegNet

DFP-Unet

Residual-Unet

Resnet101-U-Net

Segmentation Methods

Significance Threshold (p=0.05)

Paired t-test (Dice)

Paired t-test (Jaccard)

Wilcoxon Signed-Rank (Dice)

Wilcoxon Signed-Rank (Jaccard)

DeepLab Pre-trained

LinkNet Cascaded dual-scale LinkNet

SegNet VGG16

FIGURE 9. Statistical significance analysis comparing the proposed method with existing approaches. The figure displays p-values from paired t-tests and Wilcoxon signed-rank tests, demonstrating that all comparisons are statistically significant (p < 0.05).

TABLE 5. Computational efficiency comparison of U-Net, M-U-Net, and ResUNet50.

Methods Parameters

U-Net

34,513,410 M-U-Net 31,047,105 ResUNet50 13,603,649

μ T (s)

39.2

35.2 8.12

ΣT (s)

DSC JI 1960 0.8759 0.8391 1760 0.9220 0.8569 406

0.9553 0.9151

ensuring its reliability and robustness for brain tumor segmentation in MRI imaging. Statistical validation strength- ens our approach's credibility, demonstrating its clinical applicability and effectiveness in medical imaging tasks.

E. COMPUTATIONAL EFFICIENCY ANALYSIS

To assess the computational efficiency of our proposed ResUNet50, we conducted a comparative analysis against U-Net and M-U-Net based on model complexity, training time, inference speed, and segmentation accuracy. This evaluation is essential for real-world applications, where computational constraints often limit the feasibility of deep learning models in clinical settings.

Table 5 presents a detailed comparison of the three models. ResUNet50 exhibits a significantly lower parameter count of 13.6 million, compared to 34.5 million for U-Net and 31.0 million for M-U-Net. The reduced number of parameters translates into a substantial decrease in compu- tational demands, allowing for more efficient model training and faster inference. The training time for ResUNet50 is 406 seconds, which is approximately 4.8 times faster than U-Net (1960 seconds) and 4.3 times faster than M-U-Net (1760 seconds). This efficiency highlights the advantage of using ResUNet50 in resource-constrained environments where computational time is a critical factor.

In addition to training time, inference speed plays a crucial role in real-time applications such as brain tumor segmentation. The average processing time per step for ResUNet50 is 8.12 seconds, significantly outperforming U-Net (39.2 seconds) and M-U-Net (35.2 seconds). This substantial reduction in inference time underscores the practicality of ResUNet50 for clinical deployment, where rapid decision-making is essential. Faster inference allows for near real-time tumor segmentation, which is highly beneficial

VOLUME 13, 2025

for radiologists and neurosurgeons in guiding treatment strategies.

Despite its lower computational burden, ResUNet50 maintains superior segmentation accuracy. The Dice Sim- ilarity Coefficient (DSC) and Jaccard Index (JI) serve as key metrics for evaluating segmentation performance. ResUNet50 achieves a DSC of 0.9553, outperforming M- U-Net (0.9220) and U-Net (0.8759). Similarly, the Jaccard Index for ResUNet50 is 0.9151, exceeding the scores of M- U-Net (0.8569) and U-Net (0.8391). These improvements demonstrate that ResUNet50 not only accelerates process- ing but also enhances the precision of tumor boundary delineation. The significant performance boost is attributed to the incorporation of residual connections and optimized feature extraction, which allow for better retention of spatial information while reducing overfitting.

The combination of lower computational complexity, faster inference time, and superior segmentation accuracy makes ResUNet50 an ideal model for clinical applications. In practical medical scenarios, deploying deep learning mod- els on standard hardware is a challenge, and models with high computational requirements may be impractical. ResUNet50 addresses this challenge by reducing the computational load without sacrificing performance, thereby ensuring that brain tumor segmentation can be performed efficiently on accessible hardware.

In conclusion, the comparison in Table 5 confirms that ResUNet50 significantly outperforms both U-Net and M-U-Net in terms of computational efficiency and segmen- tation accuracy. With 4.8 times faster training time and 4.8 times lower inference time than U-Net while maintaining a higher DSC and Jaccard Index, ResUNet50 presents a compelling solution for real-time and clinically viable brain tumor segmentation. These findings further emphasize the practical advantages of our proposed method, demonstrating its potential for seamless integration into medical imaging workflows.

F. EVALUATION ON BRATS 2018 FOR GENERALIZATION To assess the generalization capability of the proposed ResUNet50 model, we evaluated its performance on the BraTS 2018 dataset. The experiments were conducted using all MRI modalities, and the segmentation was focused on Whole Tumor (WT). The evaluation includes segmentation accuracy, loss, Dice coefficient, and Jaccard index across High-Grade Glioma (HGG) and Low-Grade Glioma (LGG)

cases.

The results indicate that the proposed ResUNet50 model performs consistently across different MRI modalities, with Dice scores ranging from 0.9133 to 0.9201 for HGG cases and 0.9215 to 0.9251 for LGG cases. The highest Dice coefficient is achieved with T2-weighted images, which are commonly utilized in clinical settings for tumor delineation due to their enhanced contrast in pathological regions. The FLAIR modality also exhibits strong performance, demonstrating its suitability for capturing whole tumor

43485

IEEE Access

S. Saifullah et al.: Automatic Brain Tumor Segmentation: Advancing U-Net With ResNet50 Encoder

TABLE 6. Performance evaluation of ResUNet50 on different MRI modalities (BraTS 2018).

TABLE 8. Statistical significance testing for dice coefficient comparisons.

Modality

Accuracy

Loss HGG

Dice

Jaccard

Method

U-Net BrainSeg-Net

Paired t-test Wilcoxon Signed-Rank

0.0008

0.0006

0.0012

0.0010

FLAIR

0.9981

T1

0.9988

T1-CE

0.9977

T2

0.9989

0.0048 0.9187 0.8506 0.0030 0.9157 0.0055 0.9133 0.8414 0.0028 0.9201 0.8538 LGG

U-Net-FCN

0.0034

0.0029

0.8463

Ensemble Net

0.0021

0.0018

Cascaded Networks

0.0041

0.0038

ResU-Net

0.0056

0.0049

U-Net-prep

0.0015

0.0013

FLAIR

0.9980

T1

T1-CE T2

0.0049 0.9251 0.8621 0.9987 0.0033 0.9215 0.8562 0.9979 0.0051 0.9234 0.8587 0.9987 0.0032 0.9234 0.8593

AR-Net

0.0029

0.0025

Ensemble CA-Net

0.0071

0.0068

segmentation. These findings suggest that the pro- posed model effectively generalizes across different MRI sequences, enabling its applicability in a variety of diagnostic scenarios.

To further validate the effectiveness of the proposed model, a comparative analysis was conducted against other state-of- the-art segmentation approaches evaluated on BraTS 2018. The comparison is based on the Dice coefficient, as presented in Table 7.

TABLE 7. Comparison of dice coefficient on BraTS 2018 with other segmentation methods.

Method

Dice

Proposed Method

0.9202

U-Net

0.8440

BrainSeg-Net

0.8940

U-Net-FCN

0.8600

Ensemble Net

0.8810

Cascaded Networks

0.8778

ResU-Net

0.8670

U-Net-prep

0.9000

AR-Net

0.8839

Ensemble CA-Net

0.8762

The results highlight that the proposed method achieves the highest Dice coefficient of 0.9202, surpassing other approaches. Compared to the standard U-Net, which attains a Dice score of 0.8440, the proposed method shows a considerable improvement, demonstrating the advantage of residual learning and optimized skip connections. BrainSeg- Net and U-Net-prep achieve competitive performance with Dice scores of 0.8940 and 0.9000, respectively, but still fall short of the proposed method. Ensemble-based approaches such as Ensemble Net and AR-Net show improved segmenta- tion accuracy but remain inferior to the proposed ResUNet50 model.

To statistically validate the observed improvements, sig- nificance tests were conducted using a paired t-test and Wilcoxon signed-rank test. This evaluated similar with the Section IV-D.

The statistical analysis confirms that the improvements in Dice coefficient achieved by the proposed method are statistically significant, as all p-values for the paired t-test and Wilcoxon signed-rank test are below 0.05. The lowest p-values are observed in comparisons with

43486

U-Net and BrainSeg-Net, indicating substantial performance differences. The results validate that the proposed ResUNet50 model significantly outperforms prior methods, ensuring robust generalization across different tumor types and imaging conditions.

The high Dice scores across different MRI modalities, along with statistically significant improvements over prior approaches, confirm that the proposed method provides a reliable and effective solution for brain tumor segmentation in BraTS 2018. The combination of residual learning and optimized skip connections contributes to superior segmen- tation accuracy while maintaining computational efficiency. These findings highlight the potential of the proposed model for broader applications in automated medical image segmentation.

V. CONCLUSION

This study proposed a ResUNet50-based approach for brain tumor segmentation, integrating U-Net with ResNet50 as the encoder to enhance accuracy and robustness. The proposed method consistently outperformed state-of-the-art models, achieving Dice coefficients above 0.95 and Jaccard indices exceeding 0.91. Evaluations across multiple metrics, including Pixel Accuracy, Precision, Recall, F1 Score, and Mean IoU, further confirmed its superior segmentation capability.

To assess generalization, we evaluated ResUNet50 on the BraTS 2018 dataset for Whole Tumor (WT) segmentation across multiple MRI modalities (FLAIR, T1, T1-CE, and T2). The model demonstrated high segmentation performance on both High-Grade Glioma (HGG) and Low-Grade Glioma (LGG), reinforcing its effectiveness across diverse imaging conditions. Comparative analysis confirmed its superiority, and statistical significance tests (Paired t-tests and Wilcoxon Signed-Rank Tests, p < 0.05) validated the improvements over existing models. Additionally, ResUNet50 reduced parameters by over 60% and accelerated inference speed by 4.8× compared to U-Net, enhancing its suitability for clinical deployment.

Future work will address the remaining challenges in seg- mentation accuracy and model interpretability. One approach involves integrating ensemble learning techniques [79], such as model averaging and stacked generalization, to improve segmentation robustness across diverse MRI scans. Addi- tionally, bio-inspired metaheuristic optimization [80], [81]

VOLUME 13, 2025

S. Saifullah et al.: Automatic Brain Tumor Segmentation: Advancing U-Net With ResNet50 Encoder

IEEE Access

will be explored for automated hyperparameter tuning, further enhancing model adaptability. Another priority will be validating ResUNet50 on multi-center MRI datasets to evaluate its robustness across imaging protocols and scanner variations. To enhance interpretability, explainable AI (XAI) techniques such as Grad-CAM and saliency maps will be incorporated, allowing clinicians to understand model decisions and increase trust in automated segmentation systems. These advancements will enhance the model's clinical viability and interpretability. This study provides a highly accurate, efficient, and generalizable framework for brain tumor segmentation, contributing to the advancement of AI-driven medical imaging technologies in neuro-oncology.

REFERENCES

[1] S. L. Fernandes, U. J. Tanik, V. Rajinikanth, and K. A. Karthik, "A reliable framework for accurate brain image examination and treatment planning based on early diagnosis support for clinicians," Neural Comput. Appl., vol. 32, no. 20, pp. 15897–15908, Oct. 2020.

[2] A. Batool and Y.-C. Byun, "Brain tumor detection with integrating traditional and computational intelligence approaches across diverse imaging modalities-Challenges and future directions," Comput. Biol. Med., vol. 175, Jun. 2024, Art. no. 108412.

[3] S. Solanki, U. P. Singh, S. S. Chouhan, and S. Jain, "A systematic analysis of magnetic resonance images and deep learning methods used for diagnosis of brain tumor," Multimedia Tools Appl., vol. 83, no. 8, pp. 23929-23966, Aug. 2023.

[4] Y. Hu, A. Huang, and R. Xu, “HAB-net: Hierarchical asymmetric convo- lution and boundary enhancement network for brain tumor segmentation," IET Image Process., vol. 18, no. 7, pp. 1809–1822, May 2024. [5] S. Saifullah and R. Dreüewski, “Redefining brain tumor segmentation: A cutting-edge convolutional neural networks-transfer learning approach,” Int. J. Electr. Comput. Eng. (IJECE), vol. 14, no. 3, p. 2583, Jun. 2024. [6] K. V. Hoebel, C. P. Bridge, A. Kim, E. R. Gerstner, I. K. Ly, F. Deng, M. N. DeSalvo, J. Dietrich, R. Huang, S. Y. Huang, S. R. Pomerantz, S. Vagvala, B. R. Rosen, and J. Kalpathy-Cramer, "Not without context- A multiple methods study on evaluation and correction of automated brain tumor segmentations by experts," Academic Radiol., vol. 31, no. 4, pp. 1572-1582, Apr. 2024.

[7] G. Mohan and M. M. Subashini, "MRI based medical image analysis: Survey on brain tumor grade classification," Biomed. Signal Process. Control, vol. 39, pp. 139–161, Jan. 2018.

[8] X. Liu, L. Song, S. Liu, and Y. Zhang, "A review of deep-learning- based medical image segmentation methods," Sustainability, vol. 13, no. 3, p. 1224, Jan. 2021.

[9] M. A. Abdou, "Literature review: Efficient deep neural networks techniques for medical image analysis,” Neural Comput. Appl., vol. 34, no. 8, pp. 5791-5812, Apr. 2022.

[10] S. K. Natarajan, S. Jayanthi, S. K. Mathivanan, H. Rajadurai, M. B. B. A. Malar, and M. A. Shah, “Exploring fetal brain tumor glioblastoma symptom verification with self organizing maps and vulnerability data analysis,” Sci. Rep., vol. 14, no. 1, p. 8738, Apr. 2024. [11] S. Mushtaq, T. S. Walia, and A. Roy, "Deep learning-based segmentation of MRI images: Concepts, challenges, deep learning architectures, and future directions," in Intelligent Signal Processing and RF Energy Harvesting for State of Art 5G and B5G Networks, J. A. Sheikh, T. Khan, and B. K. Kanaujia, Eds., Singapore: Springer, 2024, pp. 167–178. [12] S. Saifullah, A. P. Suryotomo, R. Dreüewski, R. Tanone, and T. Tundo, "Optimizing brain tumor segmentation through CNN U-Net with CLAHE- HE image enhancement,” in Proc. 1st Int. Conf. Adv. Informat. Intell. Inf. Syst. (ICAI3S), Jan. 2024, pp. 90-101.

[13] S. Saifullah, A. Pranolo, and R. Dreüewski, "Comparative analysis of

image enhancement techniques for braintumor segmentation: Contrast, histogram, and hybrid approaches,” in Proc. E3S Web Conf., vol. 501, Jan. 2024, p. 01020.

[14] Z. Wang, Y. Zou, H. Chen, P. X. Liu, and J. Chen, “Multi-scale features and attention guided for brain tumor segmentation," J. Vis. Commun. Image Represent., vol. 100, Apr. 2024, Art. no. 104141.

VOLUME 13, 2025

[15] S. Srivastava, A. Vidyarthi, and S. Jain, “Analytical study of the encoder- decoder models for ultrasound image segmentation," Service Oriented Comput. Appl., vol. 18, no. 1, pp. 81-100, Mar. 2024.

[16] F. Yan, Y. Cao, D. An, P. Liu, X. Liao, and B. Yu, "DAUnet: A U-shaped network combining deep supervision and attention for brain tumor segmentation," Knowl.-Based Syst., vol. 285, Dec. 2023, Art. no. 111348.

[17] M. A.-A. Al-Murshidawy and O. Al-Shamma, "A review of deep learning models (U-net architectures) for segmenting brain tumors," Bull. Electr. Eng. Informat., vol. 13, no. 2, pp. 1015–1030, Apr. 2024.

[18] P. V. Kusuma and S. C. M. Reddy, "MRI brain tumor segmentation and classification with improved U-Net model,” Multimedia Tools Appl., vol. 84, no. 4, pp. 1671–1696, Apr. 2024.

[19] R. Yousef, S. Khan, G. Gupta, T. Siddiqui, B. M. Albahlal, S. A. Alajlan, and M. A. Haq, "U-net-based models towards optimal MR brain image segmentation," Diagnostics, vol. 13, no. 9, p. 1624, May 2023.

[20] N. S. Punn and S. Agarwal, "Modality specific U-net variants for biomedical image segmentation: A survey," Artif. Intell. Rev., vol. 55, no. 7, pp. 5845-5889, Oct. 2022.

[21] Z. Liu, L. Tong, L. Chen, Z. Jiang, F. Zhou, Q. Zhang, X. Zhang, Y. Jin, and H. Zhou, "Deep learning based brain tumor segmentation: A survey," Complex Intell. Syst., vol. 9, no. 1, pp. 1001–1026, Jul. 2022.

[22] J.-S. Long, G.-Z. Ma, E.-M. Song, and R.-C. Jin, "Learning U-Net based multi-scale features in encoding-decoding for MR image brain tissue segmentation," Sensors, vol. 21, no. 9, p. 3232, May 2021.

[23] A. M. Gab Allah, A. M. Sarhan, and N. M. Elshennawy, "Edge U- net: Brain tumor segmentation using MRI based on deep U-net model with boundary information," Expert Syst. Appl., vol. 213, Mar. 2023, Art. no. 118833.

[24] R. A. Zeineldin, M. E. Karar, J. Coburger, C. R. Wirtz, and O. Burgert, "DeepSeg: Deep neural network framework for automatic brain tumor segmentation using magnetic resonance FLAIR images," Int. J. Comput. Assist. Radiol. Surg., vol. 15, no. 6, pp. 909–920, Jun. 2020.

[25] M. K. Balwant, "A review on convolutional neural networks for brain tumor segmentation: Methods, datasets, libraries, and future directions," IRBM, vol. 43, no. 6, pp. 521–537, Dec. 2022.

[26] T. P. Raseena, J. Kumar, and S. R. Balasundaram, “Exploring the effectiveness of deep learning architectures for colorectal polyp detection: Performance analysis and insights," Social Netw. Comput. Sci., vol. 5, no. 5, p. 452, Apr. 2024.

[27] A. Hussain, S. U. Khan, I. Rida, N. Khan, and S. W. Baik, "Human centric attention with deep multiscale feature fusion framework for activity recognition in Internet of medical things," Inf. Fusion, vol. 106, Jun. 2024, Art. no. 102211.

[28] S. B. Mukadam and H. Y. Patil, "Machine learning and computer vision based methods for cancer classification: A systematic review,” Arch. Comput. Methods Eng., vol. 31, no. 5, pp. 3015–3050, Jul. 2024. [29] R. Ranjbarzadeh, P. Zarbakhsh, A. Caputo, E. B. Tirkolaee, and M. Bendechache, "Brain tumor segmentation based on optimized con- volutional neural network and improved chimp optimization algorithm," Comput. Biol. Med., vol. 168, Jan. 2024, Art. no. 107723.

[30] P. K. Ramtekkar, A. Pandey, and M. K. Pawar, "A comprehensive review of brain tumour detection mechanisms," Comput. J., vol. 67, no. 3, pp. 1126-1152, Apr. 2024.

[31] O. Ronneberger, P. Fischer, and T. Brox, "U-net: Convolutional networks for biomedical image segmentation," in Medical Image Computing and Computer-Assisted Intervention-MICCAI, N. Navab, J. Hornegger, W. M. Wells, and A. F. Frangi, Eds., Cham, Switzerland: Springer, 2015, pp. 234-241.

[32] L. Qian, C. Wen, Y. Li, Z. Hu, X. Zhou, X. Xia, and S.-H. Kim, "Multi-scale context UNet-like network with redesigned skip connections for medical image segmentation," Comput. Methods Programs Biomed., vol. 243, Jan. 2024, Art. no. 107885.

[33] S. E. Asl, M. C. Amirani, and H. Seyedarabi, "Brain tumors segmen- tation using a hybrid filtering with U-Net architecture in multimodal MRI volumes," Int. J. Inf. Technol., vol. 16, no. 2, pp. 1033–1042, Feb. 2024.

[34] S. Saifullah, R. Dreüewski, A. Yudhana, M. Wielgosz, and W. Caesarendra, "Modified U-Net with attention gate for enhanced automated brain tumor segmentation," Neural Comput. Appl., vol. 37, no. 7, pp. 5521-5558, Mar. 2025.

43487

IEEE Access

S. Saifullah et al.: Automatic Brain Tumor Segmentation: Advancing U-Net With ResNet50 Encoder

[35] L. Feng, K. Wu, Z. Pei, T. Weng, Q. Han, L. Meng, X. Qian, H. Xu, Z. Qiu, Z. Li, Y. Tian, G. Liang, and Y. Hao, "MLU-net: A multi- level lightweight U-net for medical image segmentation integrating frequency representation and MLP-based methods," IEEE Access, vol. 12, pp. 20734-20751, 2024.

[36] A. H. Nizamani, Z. Chen, A. A. Nizamani, and U. A. Bhatti, “Advance brain tumor segmentation using feature fusion methods with deep U-Net model with CNN for MRI data," J. King Saud Univ. - Comput. Inf. Sci., vol. 35, no. 9, Oct. 2023, Art. no. 101793.

[37] G. Huang, Z. Liu, G. Pleiss, L. van der Maaten, and K. Q. Weinberger, "Convolutional networks with dense connectivity,” IEEE Trans. Pattern Anal. Mach. Intell., vol. 44, no. 12, pp. 8704–8716, Dec. 2022.

[38] A. Khan, A. Sohail, U. Zahoora, and A. S. Qureshi, "A survey of the recent architectures of deep convolutional neural networks," Artif. Intell. Rev., vol. 53, no. 8, pp. 5455–5516, Dec. 2020.

[39] P. Kora, C. P. Ooi, O. Faust, U. Raghavendra, A. Gudigar, W. Y. Chan, K. Meenakshi, K. Swaraja, P. Pławiak, and U. R. Acharya, "Transfer learning techniques for medical image analysis: A review," Biocybernetics Biomed. Eng., vol. 42, no. 1, pp. 79-107, Dec. 2021.

[40] K. Swaraja, K. Meenakshi, H. B. Valiveti, and G. Karuna, "Segmentation and detection of brain tumor through optimal selection of integrated features using transfer learning," Multimedia Tools Appl., vol. 81, no. 19, pp. 27363-27395, Aug. 2022.

[41] R. Zoetmulder, E. Gavves, M. Caan, and H. Marquering, "Domain- and task-specific transfer learning for medical segmentation tasks," Comput. Methods Programs Biomed., vol. 214, Feb. 2022, Art. no. 106539. [42] X. Yu, J. Wang, Q.-Q. Hong, R. Teku, S.-H. Wang, and Y.-D. Zhang, "Transfer learning for medical images analyses: A survey," Neurocom- puting, vol. 489, pp. 230-254, Jun. 2022.

[43] T. Magadza and S. Viriri, “Deep learning for brain tumor segmentation: A

survey of state-of-the-art," J. Imag., vol. 7, no. 2, p. 19, Jan. 2021. [44] S. Saman and S. Jamjala Narayanan, "Survey on brain tumor segmentation and feature extraction of MR images,” Int. J. Multimedia Inf. Retr., vol. 8, no. 2, pp. 79-99, Jun. 2019.

[45] S. A. A. Ismael, A. Mohammed, and H. A. Hefny, "An enhanced deep learning approach for brain cancer MRI images classification using residual networks," Artif. Intell. Med., vol. 102, Dec. 2019, Art. no. 101779.

[46] T. Hussain and H. Shouno, "MAGRes-UNet: Improved medical image segmentation through a deep learning paradigm of multi-attention gated residual U-Net," IEEE Access, vol. 12, pp. 40290-40310, 2024. [47] J. Wang, J. Gao, J. Ren, Z. Luan, Z. Yu, Y. Zhao, and Y. Zhao, “DFP- ResUNet: Convolutional neural network with a dilated convolutional feature pyramid for multimodal brain tumor segmentation," Comput. Methods Programs Biomed., vol. 208, Sep. 2021, Art. no. 106208. [48] S. Mohammadi and M. Allali, “Advancing brain tumor segmentation with spectral-spatial graph neural networks,” Appl. Sci., vol. 14, no. 8, p. 3424, Apr. 2024.

[49] Y. Jalali, M. Fateh, and M. Rezvani, "VGA-net: Vessel graph based attentional U-net for retinal vessel segmentation," IET Image Process., vol. 18, no. 8, pp. 2191–2213, Apr. 2024.

[50] P. Mehta, S. Narwadkar, G. Chillarge, S. Rathi, G. Shinde, and C. Shewale, "A new approach to brain tumor detection with CNNS: Addressing the issues of standardization and generalizability,” Int. J. Intell. Syst. Appl. Eng., vol. 12, no. 8, pp. 1-13, 2023.

[51] A. Q. Wang, B. K. Karaman, H. Kim, J. Rosenthal, R. Saluja, S. I. Young, and M. R. Sabuncu, "A framework for interpretability in machine learning for medical imaging," IEEE Access, vol. 12, pp. 53277-53292, 2024.

[52] J. Cheng, W. Huang, S. Cao, R. Yang, W. Yang, Z. Yun, Z. Wang, and Q. Feng, "Enhanced performance of brain tumor classification via tumor region augmentation and partition," PLoS ONE, vol. 10, no. 10, Oct. 2015, Art. no. e0140381.

[53] M. Bailo, F. Gagliardi, N. Boari, A. Spina, M. Piloni, A. Castellano, and P. Mortini, "Meningioma and other meningeal tumors," Adv. Experim. Med. Biol., vol. 1405, pp. 73–97, Jan. 2023.

[54] N. Rasool and J. I. Bhat, “A critical review on segmentation of glioma brain tumor and prediction of overall survival," Arch. Comput. Methods Eng., vol. 2024, pp. 1-45, Oct. 2024.

[55] G. Thenuwara, J. Curtin, and F. Tian, "Advances in diagnostic tools and therapeutic approaches for gliomas: A comprehensive review," Sensors, vol. 23, no. 24, p. 9842, Dec. 2023.

43488

[56] I. Khalil and M. I. Hossain, "Treatment-resistant cushing disease and acromegaly in a young woman: A case of functional pituitary macroadenoma," Radiol. Case Rep., vol. 20, no. 4, pp. 2013–2019, Apr. 2025.

[57] B. H. Menze, A. Jakab, S. Bauer, J. Kalpathy-Cramer, K. Farahani, and J. Kirby, “The multimodal brain tumor image segmentation benchmark (BRATS)," IEEE Trans. Med. Imag., vol. 34, no. 10, pp. 1993–2024, Oct. 2015.

[58] S. Bakas, H. Akbari, A. Sotiras, M. Bilello, M. Rozycki, J. Kirby, J. Freymann, K. Farahani, and C. Davatzikos, “Advancing the cancer genome atlas glioma MRI collections with expert segmentation labels and radiomic features," Sci. Data, vol. 4, no. 1, Sep. 2017, Art. no. 170117. [59] S. Bakas et al., "Identifying the best machine learning algorithms for brain tumor segmentation, progression assessment, and overall survival prediction in the BRATS challenge," 2018, arXiv:1811.02629.

[60] S. Saifullah and R. Dreżewski, "Brain tumor segmentation using ensemble CNN-transfer learning models: DeepLabV3plus and ResNet50 approach,” in Proc. 24th Int. Conf. Comput. Sci., Malaga, Spain, Jan. 2024, pp. 340-354.

[61] S. Saifullah and R. Drezewski, "Enhanced medical image segmentation using CNN based on histogram equalization,” in Proc. 2nd Int. Conf. Appl. Artif. Intell. Comput. (ICAAIC), May 2023, pp. 121–126.

[62] Y. Xu, R. Quan, W. Xu, Y. Huang, X. Chen, and F. Liu, “Advances in medical image segmentation: A comprehensive review of traditional, deep learning and hybrid approaches," Bioengineering, vol. 11, no. 10, p. 1034, Oct. 2024.

[63] S. Saifullah and R. Dreżewski, "Improved brain tumor segmentation using modified U-Net based on particle swarm optimization image enhance- ment,” in Proc. Genetic Evol. Comput. Conf. (GECCO Companion), Melbourne, VIC, Australia, Jul. 2024, pp. 611–614.

[64] S. Saifullah and R. Dreżewski, "Automatic brain tumor segmentation using convolutional neural networks: U-Net framework with PSO-tuned hyperparameters," in Proc. 18th Int. Conf. Parallel Problem Solving Nature (PPSN), Hagenberg, Austria. Berlin, Germany: Springer, Jan. 2024, pp. 333-351.

[65] Y. Hu, L. Deng, Y. Wu, M. Yao, and G. Li, “Advancing spiking neural networks toward deep residual learning," IEEE Trans. Neural Netw. Learn. Syst., vol. 36, no. 2, pp. 2353-2367, Feb. 2024.

[66] M. Dabass, J. Dabass, S. Vashisth, and R. Vig, “A hybrid U-Net model with attention and advanced convolutional learning modules for simultaneous gland segmentation and cancer grade prediction in colorectal histopathological images," Intell.-Based Med., vol. 7, Jan. 2023, Art. no. 100094.

[67] Y. Chen, L. Wang, B. Ding, J. Shi, T. Wen, J. Huang, and Y. Ye, “Automated Alzheimer's disease classification using deep learning models with soft- NMS and improved ResNet50 integration," J. Radiat. Res. Appl. Sci., vol. 17, no. 1, p. 100782, 2024.

[68] F. J. D. Pernas, M. Martínez-Zarzuela, M. Antón-Rodríguez, and D. González-Ortega, "A deep learning approach for brain tumor classifi- cation and segmentation using a multiscale convolutional neural network,” Healthcare, vol. 9, no. 2, p. 153, Feb. 2021.

[69] B. V. Isunuri and J. Kakarla, "Fast brain tumour segmentation using optimized U-Net and adaptive thresholding," Automatika, vol. 61, no. 3, pp. 352-360, May 2020.

[70] P. Razzaghi, K. Abbasi, M. Shirazi, and S. Rashidi, "Multimodal brain tumor detection using multimodal deep transfer learning," Appl. Soft Comput., vol. 129, Sep. 2022, Art. no. 109631.

[71] S. B. Kumar, R. Panda, and S. Agrawal, "Brain magnetic resonance image tumor detection and segmentation using edgeless active contour," in Proc. 11th Int. Conf. Comput., Commun. Netw. Technol. (ICCCNT), Jul. 2020, Pp. 1-7. [72] S. Tripathi, A. Verma, and N. Sharma, "Automatic segmentation of

brain tumour in MR images using an enhanced deep learning approach,” Comput. Methods Biomechanics Biomed. Eng., Imag. Visualizat., vol. 9, no. 2, pp. 121-130, Sep. 2020.

[73] W. Huang and J. Wang, "Automatic segmentation of brain tumors based on DFP-UNet," in Proc. IEEE 6th Inf. Technol. Mechatronics Eng. Conf. (ITOEC), vol. 6, Mar. 2022, pp. 1304-1307.

[74] A. K. Sahoo, P. Parida, K. Muralibabu, and S. Dash, “Efficient simultaneous segmentation and classification of brain tumors from MRI scans using deep learning," Biocybernetics Biomed. Eng., vol. 43, no. 3, pp. 616-633, Jul. 2023.

VOLUME 13, 2025

S. Saifullah et al.: Automatic Brain Tumor Segmentation: Advancing U-Net With ResNet50 Encoder

IEEE Access

[75] A. Ingle, M. M. Roja, M. Sankhe, and D. Patkar, “Efficient segmentation

and classification of the tumor using improved encoder-decoder architec- ture in brain MRI images,” Int. J. Electr. Comput. Eng. Syst., vol. 13, no. 8, pp. 643-651, Nov. 2022.

[76] C. Öksüz, O. Urhan, and M. K. Güllü, “A lightweight deep model for brain tumor segmentation," in Proc. 29th Signal Process. Commun. Appl. Conf. (SIU), Jun. 2021, pp. 1–4.

[77] Z. Sobhaninia, S. Rezaei, N. Karimi, A. Emami, and S. Samavi, "Brain tumor segmentation by cascaded deep neural networks using multiple image scales," in Proc. 28th Iranian Conf. Electr. Eng. (ICEE), Aug. 2020, pp. 1-4.

[78] A. Rehman, S. Naz, U. Naseem, I. Razzak, and I. A. Hameed, "Deep auto encoder-decoder framework for semantic segmentation of brain Tumor," Aust. J. Intell. Inf. Process. Syst., vol. 15, pp. 53–60, Jan. 2019. [79] W. K. Cheung, A. Pakzad, N. Moğulkoç, S. Needleman, B. Rangelov, E. Gudmundsson, A. Zhao, M. Abbas, D. McLaverty, D. Asimakopoulos, R. Chapman, R. Savaş, S. M. Janes, Y. Hu, D. C. Alexander, J. R. Hurst, and J. Jacob, "Interpolation-split: A data-centric deep learning approach with big interpolated data to boost airway segmentation performance," J. Big Data, vol. 11, no. 1, p. 104, Aug. 2024.

[80] R. Drezewski, "A co-evolutionary multi-agent system for multi-modal function optimization,” in Proc. 4th Int. Conf. Comput. Sci. (ICCS), vol. 3038, Krakw, Poland, M. Bubak, G. D. van Albada, P. M. A. Sloot, and J. J. Dongarra, Eds., Berlin, Germany: Springer, 2004, pp. 654–661. [81] R. Drezewski and L. Siwik, "Co-evolutionary multi-agent system with predator-prey mechanism for multi-objective optimization,” in Proc. 8th Int. Conf. Adapt. Natural Comput. Algorithms (ICANNGA), vol. 4431, Warsaw, Poland. Berlin, Germany: Springer, 2007, pp. 67–76.

SHOFFAN SAIFULLAH received the bachelor's degree in informatics engineering from Univer- sitas Teknologi Yogyakarta, Indonesia, in 2015, and the master's degree in computer science from Universitas Ahmad Dahlan, Yogyakarta, in 2018. He is currently pursuing the Ph.D. degree with the AGH University of Krakow, Poland, focusing on artificial intelligence (bio-inspired algorithms), image processing, and medical image analysis. He is a Lecturer with Universitas Pembangunan Nasional Veteran Yogyakarta, Indonesia. He has received grants from AGH University and UPN Veteran Yogyakarta, and he is involved in several international collaborative research projects. His research interests include image processing, computer vision, and artificial intelligence.

RAFAŁ DREŻEWSKI received the M.Sc., Ph.D., and D.Sc. (Habilitation) degrees in computer science from the AGH University of Krakow, Poland, in 1998, 2005, and 2019, respectively. Since 2019, he has been an Associate Professor with the Institute of Computer Science, AGH University of Krakow. He is the author of more than 100 articles. His research interests include bio-inspired artificial intelligence algorithms and agent-based modeling and simulation of complex and emergent phenomena.

ANTON YUDHANA received the B.Eng. degree in electrical engineering from Institut Teknologi Sepuluh Nopember, in 2001, the M.Eng. degree in electrical engineering from Universitas Gadjah Mada, Indonesia, in 2005, and the Ph.D. degree from Universiti Teknologi Malaysia, in 2011. He is currently an Associate Professor with the Electrical Engineering Department, Universitas Ahmad Dahlan, Indonesia, and the Leader of the Center for Electrical and Electronic Research and Development (CEERD). His current research interests include agriculture precision, biomedical engineering, and food engineering based on the Internet of Things.

ANDIKO PUTRO SURYOTOMO received the bachelor's and master's degrees in computer sci- ence from Universitas Gadjah Mada, Yogyakarta, Indonesia, in 2011 and 2017, respectively. He is currently a Lecturer with Universitas Pemban- gunan Nasional Veteran Yogyakarta, Indonesia. His research interests include digital image pro- cessing, machine learning, and high-performance computing.

VOLUME 13, 2025

43489